RAG Definition — Agnostic Requirements

Receive • Augment • Generate — mission‑critical behaviors across layers

Project: AMU / Nikola
Author: Steven Westermire (Maddog / Gunny)
Co-Author: M365 Copilot (Microsoft)
Version: v0.1.1
Date: January 16, 2026 08:17 AM PST

## Change Log

| Date | Version | Author | Notes |
| --- | --- | --- | --- |
| January 16, 2026 08:17 AM PST | v0.1.1 | Steven Westermire (Maddog / Gunny) | Propagation-aligned: Receive = Acquire/Validate/Decide with Admit/Quarantine/Reject; clarified acceptance and triage language. |
| January 16, 2026 08:17 AM PST | v0.1 | Steven Westermire (Maddog / Gunny) | Initial issue: agnostic RAG definitions and acceptance criteria. |

## Mission Statement

We shall RECEIVE data. We shall AUGMENT data. We shall GENERATE.

Objective: maintain positive control, preserve safety, and deliver mission effects — continuously, verifiably, and without excuse.

## Core Principle (Agnostic)

RAG is layer‑independent. These requirements apply to any edge/controller (e.g., sensors/actuators), any autonomy/transport node, and any coordination station.

## R — RECEIVE (Acquire → Validate → Decide)

• Acquire: Input arrives at a boundary (link/port/queue) and is captured exactly once.

• Validate: Check identity, schema/version, sequence/ordering, timestamp sanity, and state gating for commands. (When security is enabled, verify integrity/auth.)

• Decide: One of three outcomes — Admit (good), Quarantine (incomplete/out‑of‑order), Reject (bad/unsafe). Reason is recorded for non‑admit outcomes.

### Receive — Acceptance Criteria

Every inbound item has a Receive outcome (Admit/Quarantine/Reject).

Non‑admit outcomes include a human‑readable reason (e.g., schema, seq, ts, state).

Receive counters exist and are visible per type and per layer.

Quarantine policy is defined (hold duration, release conditions).

Rejected items create no side‑effects beyond logging/alerts.

## A — AUGMENT (Normalize → Fuse → Annotate)

• Normalize: Convert to canonical units/fields per schema with explicit versions.

• Fuse: Combine inputs (e.g., pose/health) into coherent state as required by the application domain.

• Annotate: Attach context (mission/geofence), link metrics, and derived flags/events.

### Augment — Acceptance Criteria

Normalization rules are documented and testable (before→after examples).

Fusion logic produces deterministic fields with versioned definitions.

Derived state changes emit discrete events with timestamps and identifiers.

## G — GENERATE (Decide → Transmit → Confirm)

• Decide: Determine which product to emit (status, command, alert) based on state/policy.

• Transmit: Use the contract envelope, with sequence and acknowledgment policy.

• Confirm: Capture acknowledgments/outcomes and apply retries/backoff or escalation rules.

### Generate — Acceptance Criteria

Each generated item is traceable: who/what/when/to‑whom/result.

ACK policy is explicit: required vs optional per type; timeouts and retries documented.

No item is generated from data that did not pass Receive (Admit).

## Cross‑Cutting Requirements (Agnostic)

Layer boundaries are respected; lower layers are not addressed directly by higher layers except via defined contracts.

Safety precedence is defined (e.g., E‑STOP overrides all).

Versioning is explicit (protocol_version, schema_version) with compatibility policy.

Time coherence is maintained; record both frame time and receive time.

Observability: counters, events, and logs sufficient for a 2‑minute triage.

Reliability: backoff, dedupe, and degraded‑mode behaviors are specified.

Security: pluggable posture (OFF for lab is permissible); when ON, integrity/auth and replay guards are enforced.

## Measures of Performance (MOP) & Effectiveness (MOE)

• MOP (mechanics): Receive‑Admit rate, ACK success, cadence variance, data freshness.

• MOE (mission): completion within tolerances, time‑to‑SAFE on faults, operator time‑to‑truth.

## Triage Quick Reference (Receive Outcomes)

Reject — Schema/Version: Check contract versions and field names.

Reject — Sequence: Investigate gaps/duplicates; verify sender seq policy.

Reject — Timestamp: Verify clock skew and frame freshness.

Reject — State: Command illegal in current state; review state‑gating.

Quarantine — Out‑of‑Order/Partial: Hold until gap closes or timeout, then decide.

## Glossary

RAG: Receive • Augment • Generate — layered, auditable process.

Admit/Quarantine/Reject: Receive decisions for inbound items.

Schema Version: Versioned definition of fields/units/formats in a payload.

ACK Policy: Which message types require acknowledgments and how timeouts/retries apply.

Degraded Mode: Behavior under partial failure (reduced rates, SAFE states, etc.).